EPC User’s Guide

| Editor: | EURECOM |
| --- | --- |
| Deliverable nature: | Public |
| Due date: | July, 2015 |
| Delivery date: | July, 2015 |
| Version: | 0.7 |
| Total number of pages: | 37 |
| Reviewed by: |  |
| Keywords: | EPC, LTE, MME, S-GW, HSS, S1AP, DIAMETER |

Abstract

The deliverable presents the EPC developed by EURECOM.

The document presents the deployment scenarios of the EPC, its configuration, installation testing and running.

List of authors

| Company | Authors |
| --- | --- |
| EURECOM | Christian BONNET, Lionel GAUTHIER, Rohit GUPTA, Florian KALTENBERGER, Raymond KNOPP, Navid NIKAIEN, Cedric ROUX. |
| OPEN CELLS | Laurent THOMAS. |

History

| Modified by | Date | Version | Comments |
| --- | --- | --- | --- |
| Lionel GAUTHIER | 09/06/2015 | 0.1 | Initial Draft |
| Lionel GAUTHIER | 11/06/2015 | 0.2 | Minor corrections |
| Lionel GAUTHIER | 20/07/2015 | 0.3 | Changes in dir and file names for git repo |
| Lionel GAUTHIER | 29/07/2015 | 0.4 | Minor corrections (build, get sources from git) |
| Lionel GAUTHIER | 09/09/2015 | 0.5 | Minor corrections (build_epc script) |
| Lionel GAUTHIER | 21/09/2015 | 0.6 | History, GTP-U installation for EPC |
| Lionel GAUTHIER | 14/12/2015 | 0.7 | Removed epc config file when eNB and EPC are co-located, added description of S1AP scenario replay |

Table of Contents

History	3

Table of Contents	4

List of Figures	6

List of tables	7

Abbreviations	8

1	Introduction	9

1.1	Overview	9

1.2	Deployment scenarios	9

1.2.1	Separate EPC platform	9

1.2.2	All in one EPC platform	10

2	EPC Installation	11

2.1	Operating system	11

2.2	EPC source code	11

2.2.1	Get the code without login	11

2.2.2	Get the code with login (contributors)	11

2.3	Additional software, initial steps.	12

2.3.1	Mysql server installation details	12

2.3.2	Phpmyadmin installation details	12

3	EPC Configuration	15

3.1	MME_GW	15

3.1.1	Fully Qualified Domain name	15

3.1.2	Configuration files	15

3.2	MME	16

3.3	SP_GW	16

3.4	MME configuration content	16

3.4.1	GUMMEI section	16

3.4.2	SCTP section	17

3.4.3	S1AP section	17

3.4.4	S6A section	17

3.4.5	NAS section	17

3.4.6	INTERTASK_INTERFACE section	17

3.4.7	Network interfaces section	17

3.5	S-GW configuration content	18

3.6	P-GW configuration content	19

3.6.1	Main section	19

3.6.2	Network interfaces section	19

3.6.3	IP Address Pool section	19

3.7	HSS	20

3.7.1	Fully Qualified Domain name	20

3.7.2	Configuration files	20

3.7.3	HSS database content	23

3.7.4	Configuring your HSS	25

Adding your MME	25

Adding a user	25

4	Building and running	27

4.1	MME_GW.	27

4.1.1	Configuration files	27

4.1.2	Building EPC	27

4.1.3	Running EPC	27

5	Supported scenarios in EPC	28

5.1	E-UTRAN Initial attach	28

5.1.1	Attach with IMSI	28

5.1.2	Attach with GUTI	28

5.2	Tracking Area Update procedures	28

5.3	Routing Area Update procedures	28

5.4	Service Request procedures	28

5.4.1	UE triggered Service Request	28

5.4.2	Network triggered Service Request	28

5.5	S1 Release procedure	28

5.6	GUTI Reallocation procedure	28

5.7	Detach procedure	28

5.7.1	UE-Initiated Detach procedure for E-UTRAN	28

5.7.2	MME-Initiated Detach procedure for E-UTRAN	28

5.7.3	HSS-Initiated Detach procedure for E-UTRAN	28

5.8	HSS User Profile management function procedure	28

5.9	Bearer deactivation	28

5.9.1	PDN GW initiated bearer deactivation	28

5.9.2	MME initiated Dedicated Bearer Deactivation	28

5.10	Intra E-UTRAN handover	29

6	Annex A: Tools for observing, debugging.	30

6.1	Itti_analyzer	30

6.1.1	Installation	30

6.1.2	Execution	30

6.2	Wireshark/tshark	31

6.3	Mscgen	31

6.4	S1AP scenario replay	33

6.4.1	Overall process	33

6.4.2	Flowchart of step1: Network trace capture on S1-C	33

6.4.3	Build a generic scenario	36

6.4.4	Replay a S1AP generic scenario	37

List of Figures

Figure 1 EPC overview	9

Figure 2 EPC Deployment in MME SP-GW	9

Figure 3 EPC Deployment in MME_GW	10

Figure 4 Mysql installation root password	12

Figure 5 Phpmyadmin installation conf DB	13

Figure 6 Phpmyadmin installation DB password	13

Figure 7 Phpmyadmin installation app password	13

Figure 8 Phpmyadmin installation web server selection	14

Figure 9 MME_GW configuration files generation	15

Figure 10 HSS configuration files generation	20

Figure 11 ITTI Analyzer main window	30

Figure 12 ITTI Analizer select filter menu	31

Figure 13 Mscgen output example	32

Figure 14 Workflow of scenario replay	33

Figure 15 Flowchart of scenario capture	34

Figure 16 Build scenario detailed operations	36

List of tables

Table 1 MME configuration main section	16

Table 2 MME configuration subsection GUMMEI	16

Table 3 MME configuration subsection SCTP	17

Table 4 MME configuration subsection S1AP	17

Table 5 MME configuration subsection S6a	17

Table 6 MME configuration subsection NAS	17

Table 7 MME configuration subsection ITTI	17

Table 8 MME configuration subsection Network Interfaces	18

Table 9 S-GW configuration main section	18

Table 10 P-GW configuration main section	19

Table 11 P-GW configuration subsection Network Interfaces	19

Table 12 P-GW configuration subsection IP Address Pool Selection	19

Table 13 SQL Table structure mmeidentity	23

Table 14 SQL Table structure pdn	24

Table 15 SQL Table structure users	25

Abbreviations

3GPP 		Third Generation Partnership Project.

APN		Access Point Name.

CIDR	Classless Inter-Domain Routing.

eNB	e Node B.

EPC		Evolved Packet Core.

EPS		Evolved Packet System.

FQDN		Fully qualified domain name.

HSS		Home Subscriber Server.

IMEI 	International Mobile Station Equipment Identity.

IMEISV	International Mobile Station Equipment Identity Software

Version.

LTE		Long Term Evolution.

MME		Mobility Management Entity.

MSISDN		Mobile Station International Subscriber Directory Number.

NW		Network.

P-GW		PDN Gateway, Packet Data Network Gateway.

PDN		Packet Data Network.

QoS		Quality of Service.

SCTP		Stream Control Transmission Protocol.

S-GW		Serving Gateway.

SIM 	Subscriber Identity Module.

TCP	Transmission Control Protocol.

USIM 	Universal Subscriber Identity Module.

Introduction

Overview

The EURECOM EPC is a bundle of software components that provides the MME, S+P-GW, HSS functions of the LTE core EPC architecture (http://www.3gpp.org/DynaReport/23002.htm).

Actually the SGW and the PGW are merged together, there is no S5 or S8 interface between the two functional entities.

Figure 1 EPC overview

Deployment scenarios

Two deployment scenarios are considered with the EURECOM EPC.

Separate EPC platform

Actually this deployment scenario is under development and cannot be demonstrated yet.

Figure 2 EPC Deployment in MME SP-GW

All in one EPC platform

The following picture depicts a EURECOM EPC providing MME and GW functions, and interact with the EURECOM HSS. In this deployment scenario, the S11 interface is virtual in the sense that S11 messages do not go through the network layer but through an inter-task interface message passing middleware (ITTI).

Figure 3 EPC Deployment in MME_GW

The EPC can be deployed on the same EURECOM eNB host or on its own host.

The HSS can be deployed on the same EPC host, EURECOM eNB host or on its own host. Any combination of deployment with one, two or three host(s) is possible with the EURECOM eNB.

If a third party eNB is used, then it is preferable to run the EPC and HSS on one or two other hosts, indifferently.

EPC Installation

Operating system

The EPC software has only been tested on UBUNTU 14.04x64, and UBUNTU 14.10x64 LINUX distributions on Intel x86 64 bits platforms.

If you want to try another LINUX distribution, it is mandatory to have a 64 bits LINUX distribution.

Important!

In this document OPENAIRCN_DIR is the path to the openair-cn working directory.

EPC source code

The OpenAirInterface core network software can be obtained from our git server. You will need a git client to get the sources.

If git is not installed on your computer, execute in a shell the following command (Ubuntu):

user@host:~ sudo apt-get install git

Configure git with your name/email address (only important if you are developer and want to checkin code to Git):

git config --global user.name "Your Name"

git config --global user.email "Your email address"

Add a certificate from gitlab.eurecom.fr to your Ubuntu 14.04 installation (you need to be root user):

echo -n | openssl s_client -showcerts -connect gitlab.eurecom.fr:443 2>/dev/null | sed -ne '/-BEGIN CERTIFICATE-/,/-END CERTIFICATE-/p' >> /etc/ssl/certs/ca-certificates.crt

Get the code without login

In order to checkout the Git repository (for OAI Users without login to gitlab server)

user@host:~ git clone https://gitlab.eurecom.fr/oai/openair-cn.git

Get the code with login (contributors)

In order to check out the Git repository (for OAI Developers/admins with login to gitlab server)

Please send an email to openair_tech@eurecom.fr to be added to the repository as a developer (only important for users who want to commit code to the repository). If you do not have account on gitlab.eurecom.fr, please register yourself to gitlab.eurecom.fr.

Checkout with using ssh keys:

You will need to put your ssh keys in https://gitlab.eurecom.fr/profile/keys to access to the git repo. Once that is done, checkout the git repository using:

git clone git@gitlab.eurecom.fr:oai/openair-cn.git

Checkout with user name/password prompt:

git clone https://YOUR_USERNAME@gitlab.eurecom.fr/oai/openair-cn.git

Additional software, initial steps.

Some software installations have to be done prior to build the EURECOM EPC and the EURECOM HSS.

In OPENAIRCN_DIR/SCRIPTS directory, execute the following command:

user@host:~/openair-cn/SCRIPTS$ ./build_epc -i

This command will update the software source list of your Ubuntu installation. It will install miscellaneous software packages, mainly an openair-cn version (patched) of freeDiameter, an openair version (patched) of asn1c,  and particularly mysql-server and phpmyadmin software, which steps are described below.

This command will also install the GTP-U kernel part of the GTP-U protocol layer of the S-GW. For licensing reasons, this code is located in another git repository: git@gitlab.eurecom.fr:oai/xtables-addons-oai.git.

This kernel module requires the installation of a kernel version greater or equal to 3.19, this constraint is handled in the command build_epc -I, or build_epc -j (install only the xtables-addons-oai.git software): you are prompted if necessary to install a linux kernel and/or reboot your operating system on another linux kernel.

Mysql server installation details

Enter here the root password of your host.

Figure 4 Mysql installation root password

The mysql-server installation process ends here.

Phpmyadmin installation details

You should prefer the easiest way

Figure 5 Phpmyadmin installation conf DB

Enter here the root password of your host:

Figure 6 Phpmyadmin installation DB password

Accordingly with the content of openair-cn configuration files, please, enter here admin

Figure 7 Phpmyadmin installation app password

Choose the web server that has to be configured: Apache.

Figure 8 Phpmyadmin installation web server selection

EPC Configuration

MME_GW

Fully Qualified Domain name

A FQDN has to be set for the MME_GW (freeDiameter constraint ACL about this may not exist anymore, to be checked). An easy way to do that is to fill this FQDN in the /etc/hosts file.

Example:

yang@yang:$ cat /etc/hosts
127.0.0.1   localhost
127.0.1.1   yang.openair4G.eur  yang
...

192.168.12.175 yin.openair4G.eur hss yin

...

Configuration files

Here is view of the build process of MME_GW, we can see there when and how configuration files are generated. Inputs files and parameters are on the left part of the figure, the build process is in the center part and output configuration files are on the right of the figure.

Figure 9 MME_GW configuration files generation

Configuration file epc.conf and epc.local.enb.conf:

These configuration files, since MME_GW is an aggregation of a MME, a S-GW and a P-GW, aggregate three configuration sections: a MME, a S-GW, and a P-GW configuration section.

This configuration files follow the libconfig file syntax (http://www.hyperrealm.com/libconfig).

These sections are described below.

Configuration file mme_fd.conf:

This configuration file is the input file for configuring the diameter protocol instance of the MME_GW.

MME

Empty section, will be updated when a standalone MME will be released.

SP_GW

Empty section, , will be updated when a standalone S+P-GW will be released.

MME configuration content

| Parameter | Type |  |
| --- | --- | --- |
| REALM | String | Diameter realm of the MME |
| MAXENB | Num/Integer | Maximum number of eNB that can connect to MME. |
| MAXUE | Num/Integer | For debug purpose, used to restrict the number of served UEs the MME can handle. |
| RELATIVE_CAPACITY | Num/Integer | Even though this parameter is not used by the MME for controlling the MME load balancing within a pool (at least for now), the parameter has to be forwarded to the eNB during association procedure. Values going from 0 to 255, (Default value is 15) |
| MME_STATISTIC_TIMER | Num/Integer | Displayed statistic (stdout) period. |
| EMERGENCY_ATTACH_SUPPORTED | String |  |
| UNAUTHENTICATED_IMSI_SUPPORTED | String |  |
| IP_CAPABILITY | String | Choice between IPV4, IPV4V6, IPV4ORV6 |

Table 1 MME configuration main section

GUMMEI section

| Parameter | Type |  |
| --- | --- | --- |
| MME_CODE | Array of Num/Integer | List of a maximum of 256 values can be provided. MME code range is [0..255] |
| MME_GID | Array of Num/Integer | List of maximum 65536 values. MME group id range is [0..65535] |
| TAI | Array of TAI (PLMN:TAC) | List of maximum 32 TAI. (TAI=MCC.MNC:TAC) |

Table 2 MME configuration subsection GUMMEI

SCTP section

| Parameter | Type |  |
| --- | --- | --- |
| SCTP_INSTREAMS | Num/Integer | Num streams for UE association signaling, note that stream with id =0 is reserved for non-Ue associated signaling. At least two streams should be used by the MME. (Default value=64) |
| SCTP_OUTSTREAMS | Num/Integer | Idem above |

Table 3 MME configuration subsection SCTP

S1AP section

| Parameter | Type |  |
| --- | --- | --- |
| S1AP_OUTCOME_TIMER | Num/Integer | Once an outcome is sent from MME to eNB, the MME locally starts a timer to abort the procedure and release UE context if the expected answer to this outcome is not received at the expiry of this timer. This timer is expressed in seconds. (Default value = 5 seconds) |

Table 4 MME configuration subsection S1AP

S6A section

| Parameter | Type |  |
| --- | --- | --- |
| S6A_CONF | String | S6A config file path |
| HSS_HOSTNAME | String | HSS hostname |

Table 5 MME configuration subsection S6a

NAS section

| Parameter | Type |  |
| --- | --- | --- |
| ORDERED_SUPPORTED_INTEGRITY_ALGORITHM_LIST | Array of String | Preference list in decreasing order of supported integrity algorithms, actually supported integrity algorithms are EIA0, EIA1, EIA2 |
| ORDERED_SUPPORTED_CIPHERING_ALGORITHM_LIST | Array of String | Preference list in decreasing order of supported integrity algorithms, actually supported integrity algorithms are EEA0, EEA1, EEA2 |

Table 6 MME configuration subsection NAS

INTERTASK_INTERFACE section

| Parameter | Type |  |
| --- | --- | --- |
| ITTI_QUEUE_SIZE | Num/Integer | Upper bound for the message queue size expressed in bytes (all messages exchanged by tasks have the same size). Restrict the number of messages in queues or detect a possible MME overload. |

Table 7 MME configuration subsection ITTI

Network interfaces section

| Parameter | Type |  |
| --- | --- | --- |
| MME_INTERFACE_NAME_FOR_S1_MME | String | Interface name for S1-MME (S1-C) |
| MME_IPV4_ADDRESS_FOR_S1_MME | String, CIDR | Binded address for S1-MME |
| MME_INTERFACE_NAME_FOR_S11_MME | String | Interface name for S11, “none” if S11 unused |
| MME_IPV4_ADDRESS_FOR_S11_MME | String, CIDR | Binded address for S11, (0.0.0.0/xx) if S11 unused |

Table 8 MME configuration subsection Network Interfaces

S-GW configuration content

| Parameter | Type |  |
| --- | --- | --- |
| SGW_INTERFACE_NAME_FOR_S11 | String | Interface name for S11, “none” if S11 unused |
| SGW_IPV4_ADDRESS_FOR_S11 | String, CIDR notation | Binded address for S11, (0.0.0.0/xx) if S11 unused |
| SGW_INTERFACE_NAME_FOR_S1U_S12_S4_UP | String | Interface name for S1-U |
| SGW_IPV4_ADDRESS_FOR_S1U_S12_S4_UP | String, CIDR notation | Binded address for S1-U |
| SGW_IPV4_PORT_FOR_S1U_S12_S4_UP | Num/Integer | Port number for S1-U (IANA), Should be 2152 |
| SGW_INTERFACE_NAME_FOR_S5_S8_UP | String, | Interface name for S5 or S8, “none” because unused |
| SGW_IPV4_ADDRESS_FOR_S5_S8_UP | String, CIDR notation | Binded address for S5 or S8, (0.0.0.0/xx) because unused |

Table 9 S-GW configuration main section

P-GW configuration content

Main section

| Parameter | Type |  |
| --- | --- | --- |
| DEFAULT_DNS_1_IPV4_ADDRESS | String, IPv4 dot decimal | IPv4 address of primary default DNS that can be queried by UEs |
| DEFAULT_DNS_2_IPV4_ADDRESS | String, IPv4 dot decimal | IPv4 address of secondary default DNS that can be queried by UEs |

Table 10 P-GW configuration main section

Network interfaces section

| Parameter | Type |  |
| --- | --- | --- |
| PGW_INTERFACE_NAME_FOR_S5_S8 | String | Interface name for S5 or S8, “none” because unused |
| PGW_IPV4_ADDRESS_FOR_S5_S8 | String, CIDR notation | Binded address for S5 or S8, (0.0.0.0/xx) because unused |
| PGW_INTERFACE_NAME_FOR_SGI | String | Interface name for SGi |
| PGW_IPV4_ADDRESS_FOR_SGI | String, CIDR notation | Used IPv4 address for SGi, useful if UE traffic is masqueraded. |
| PGW_MASQUERADE_SGI | String | Should outgoing UE IPv4 traffic be masqueraded (source NAT), “yes” or “no”. |

Table 11 P-GW configuration subsection Network Interfaces

IP Address Pool section

| Parameter | Type |  |
| --- | --- | --- |
| IPV4_LIST | String, CIDR notation | List of IPv4 netmasks that designate a list of available IPv4 addresses for UEs |
| IPV6_LIST | String, CIDR notation | List of IPv6 netmasks that designate a list of available IPv6 addresses for UEs |

Table 12 P-GW configuration subsection IP Address Pool Selection

HSS

Fully Qualified Domain name

A FQDN has to be set for the HSS. An easy way to do that is to fill this FQDN in the /etc/hosts file.

Example:

yin@yin:$ cat /etc/hosts
127.0.0.1   localhost
127.0.1.1   yin.openair4G.eur  yin
...

Configuration files

Here is partial view of the build process of HSS, we can see there when and how configuration files are generated. Inputs files and parameters are on the left part of the figure, the build process is in the center part and output configuration files are on the right of the figure.

Figure 10 HSS configuration files generation

Configuration file hss.conf.in:

This configuration file is the top configuration file containing all necessary parameters and links to other configuration files. This file do not need to be edited, all parameters passed to the build_hss executable and also its default parameters are substituted in the right place in this config file.

hss.conf.in content:

## MySQL mandatory options

MYSQL_server = "@MYSQL_server@";

MYSQL_user   = "@MYSQL_user@";

MYSQL_pass   = "@MYSQL_pass@";

MYSQL_db     = "@MYSQL_db@";

## HSS options

OPERATOR_key = "@OPERATOR_key@";

RANDOM = "@RANDOM_boolean@";

## Freediameter options

FD_conf = "@FREEDIAMETER_PATH@/../etc/freeDiameter/hss_fd.conf";

The following is an example of the resulting config file hss.conf:

## MySQL mandatory options

MYSQL_server = "127.0.0.1";

MYSQL_user   = "hssadmin";

MYSQL_pass   = "admin";

MYSQL_db     = "oai_db";

## HSS options

OPERATOR_key = "11111111111111111111111111111111";

RANDOM = "FALSE";

## Freediameter options

FD_conf = "/usr/lib/../etc/freeDiameter/hss_fd.conf";

Configuration file hss_fd.conf.in:

This configuration file is the input file for configuring the diameter protocol instance of the HSS.

All parameters values between ‘@’ are filled by the cmake process. These parameters are set with the help of input parameters passed to the build_hss executable, and with the help of default values set in the OPENAIRCN_DIR/BUILD/HSS/CMakeLists.txt file.

You can see here what are default values defined in OPENAIRCN_DIR/BUILD/HSS/CMakeLists.txt and set your own:

set(MYSQL_server      "127.0.0.1"      CACHE STRING  "Database server IP address")

set(MYSQL_admin       root             CACHE STRING  "Database admin login")

set(MYSQL_admin_pass  linux            CACHE STRING  "Database admin password")

set(MYSQL_user        hssadmin         CACHE STRING  "Database username login")

set(MYSQL_pass        admin            CACHE STRING  "Database username password")

set(MYSQL_db          oai_db           CACHE STRING  "Database name")

set(TRANSPORT_option  "#No_TCP"        CACHE STRING  "No_TCP or No_SCTP or comment string, FreeDiameter config option")

set(TRANSPORT_PREFER_TCP_option  "#Prefer_TCP"   CACHE STRING  "Prefer_TCP or comment string, FreeDiameter config option")

set(AppServThreads    2                CACHE STRING  "FreeDiameter AppServThreads config option")

set(OPERATOR_key      ""               CACHE STRING  "LTE operator clear text key (hex bytes) example 11111111111111111111111111111111")

set(RANDOM_boolean    "true"           CACHE STRING  "If false, random function returns always 0, else random as usual.")

set(REMOTE_PEER_WHITELIST "*.${REALM}" CACHE STRING  "Remote peer whitelist (separated by spaces), for freediameter acl.conf config file")

hss_fd.conf.in content:

# -------- Local ---------

# The first parameter in this section is Identity, which will be used to

# identify this peer in the Diameter network. The Diameter protocol mandates

# that the Identity used is a valid FQDN for the peer. This parameter can be

# omitted, in that case the framework will attempt to use system default value

# (as returned by hostname --fqdn).

Identity = "@HSS_FQDN@";

# In Diameter, all peers also belong to a Realm. If the realm is not specified,

# the framework uses the part of the Identity after the first dot.

Realm = "@REALM@";

# This parameter is mandatory, even if it is possible to disable TLS for peers

# connections. A valid certificate for this Diameter Identity is expected.

TLS_Cred = "@FREEDIAMETER_PATH@/../etc/freeDiameter/hss.cert.pem", "@FREEDIAMETER_PATH@/../etc/freeDiameter/hss.key.pem";

TLS_CA = "@FREEDIAMETER_PATH@/../etc/freeDiameter/hss.cacert.pem";

# Disable use of TCP protocol (only listen and connect in SCTP)

# Default : TCP enabled

@TRANSPORT_option@;

# This option is ignored if freeDiameter is compiled with DISABLE_SCTP option.

# Prefer TCP instead of SCTP for establishing new connections.

# This setting may be overwritten per peer in peer configuration blocs.

# Default : SCTP is attempted first.

@TRANSPORT_PREFER_TCP_option@;

# Disable use of IPv6 addresses (only IP)

# Default : IPv6 enabled

No_IPv6;

# Overwrite the number of SCTP streams. This value should be kept low,

# especially if you are using TLS over SCTP, because it consumes a lot of

# resources in that case. See tickets 19 and 27 for some additional details on

# this.

# Limit the number of SCTP streams

SCTP_streams = 3;

# By default, freeDiameter acts as a Diameter Relay Agent by forwarding all

# messages it cannot handle locally. This parameter disables this behavior.

NoRelay;

TLS_old_method;

# Number of parallel threads that will handle incoming application messages.

# This parameter may be deprecated later in favor of a dynamic number of threads

# depending on the load.

AppServThreads = @AppServThreads@;

# Specify the addresses on which to bind the listening server. This must be

# specified if the framework is unable to auto-detect these addresses, or if the

# auto-detected values are incorrect. Note that the list of addresses is sent

# in CER or CEA message, so one should pay attention to this parameter if some

# adresses should be kept hidden.

@ListenOn@;

@DIAMETER_PORT@;

@DIAMETER_SEC_PORT@;

# -------- Extensions ---------

# Uncomment (and create rtd.conf) to specify routing table for this peer.

#LoadExtension = "rt_default.fdx" : "rtd.conf";

# Uncomment (and create acl.conf) to allow incoming connections from other peers.

LoadExtension = "acl_wl.fdx" : "@FREEDIAMETER_PATH@/../etc/freeDiameter/acl.conf";

# Uncomment to display periodic state information

#LoadExtension = "dbg_monitor.fdx";

# Uncomment to enable an interactive Python interpreter session.

# (see doc/dbg_interactive.py.sample for more information)

#LoadExtension = "dbg_interactive.fdx";

# Load the RFC4005 dictionary objects

#LoadExtension = "dict_nasreq.fdx";

LoadExtension = "dict_nas_mipv6.fdx";

LoadExtension = "dict_s6a.fdx";

# Load RFC4072 dictionary objects

#LoadExtension = "dict_eap.fdx";

# Load the Diameter EAP server extension (requires diameap.conf)

#LoadExtension = "app_diameap.fdx" : "diameap.conf";

# Load the Accounting Server extension (requires app_acct.conf)

#LoadExtension = "app_acct.fdx" : "app_acct.conf";

# -------- Peers ---------

# The framework will actively attempt to establish and maintain a connection

# with the peers listed here.

# For only accepting incoming connections, see the acl_wl.fx extension.

#ConnectPeer = "ubuntu.localdomain" { ConnectTo = "127.0.0.1"; No_TLS; };

@ConnectPeer@ = "@MME_FQDN@"  { ConnectTo = "@MME_IP@"; Realm = "@REALM@"; No_IPv6; No_TLS ; port = 3870; };

Configuration file acl.conf.in:

TODO

HSS database content

SQL operations (display, update, export, etc) can be done easily with the help of phpMyAdmin, you have to open the following URL with your browser: http://yourhsshost/phpmyadmin.

Otherwise you can use any other MySQL tool, script compatible with MySQL.

Table mmeidentity:

Structure:

| Field | Type | Null | Key | Default | Extra |
| --- | --- | --- | --- | --- | --- |
| idmmeidentity | int(11) | NO | PRI | NULL | auto_increment |
| mmehost | varchar(255) | YES |  | NULL |  |
| mmerealm | varchar(200) | YES |  | NULL |  |
| UE-Reachability | tinyint(1) | NO |  | NULL |  |

Table 13 SQL Table structure mmeidentity

Column idmmeIdentity is the primary key of a MME.

Column mmehost contains the FQDN of a MME.

Column mmerealm contains the realm of a MME.

Example of content:

+---------------+------------------------+---------------+-----------------+
| idmmeidentity | mmehost                | mmerealm      | UE-Reachability |
+---------------+------------------------+---------------+-----------------+
|             2 | yang.openair4G.eur     | openair4G.eur |               0 |
|             1 | ng40-erc.openair4G.eur | openair4G.eur |               0 |
|             3 | ABEILLE.openair4G.eur  | openair4G.eur |               0 |
+---------------+------------------------+---------------+-----------------+

Table pdn:

This table contains mainly the association between a user and a APN, and its QOS parameters.

Structure:

| Field | Type | Null | Key | Default | Extra |
| --- | --- | --- | --- | --- | --- |
| id | int(11) | NO | PRI | NULL | auto_increment |
| apn | varchar(60) | NO |  | NULL |  |
| pdn_type | enum('IPv4','IPv6','IPv4v6','IPv4_or_IPv6') | NO |  | NULL |  |
| pdn_ipv4 | varchar(15) | YES |  | NULL | 0.0.0.0 |
| pdn_ipv6 | varchar(45) | YES |  | NULL | 0:0:0:0:0:0:0:0 |
| aggregate_ambr_ul | int(10) unsigned | YES |  | 50000000 |  |
| aggregate_ambr_dl | int(10) unsigned | YES |  | 100000000 |  |
| pgw_id | int(11) | NO | PRI | NULL |  |
| users_imsi | varchar(15) | NO | PRI |  |  |
| qci | tinyint(3) unsigned | NO |  | 9 |  |
| priority_level | tinyint(3) unsigned | NO |  | 15 |  |
| pre_emp_cap | enum('ENABLED','DISABLED') | YES |  | DISABLED |  |
| pre_emp_vul | enum('ENABLED','DISABLED') | YES |  | DISABLED |  |
| LIPA-Permissions | enum('LIPA-prohibited','LIPA-only','LIPA-conditional') | YES |  | LIPA-only |  |

Table 14 SQL Table structure pdn

Column id is the primary key of a pdn entry.

Column pdn_type contains the type of PDN, actually only IPv4 is supported.

Column pdn_ipv4 contains the IPv4 address of the PDN (unused).

Column pdn_ipv6 contains the IPv6 address of the PDN (unused).

Column aggregate_ambr_ul  TODO

Column aggregate_ambr_dl TODO

Column pgw_id TODO

Column users_imsi TODO

Column qci TODO

Column priority_level TODO

Column pre_emp_capability TODO

Column pre_emp_vulnerability TODO

Column LIPA_Permissions TODO

Table users

Structure:

| Field | Type | Null | Key | Default | Extra |
| --- | --- | --- | --- | --- | --- |
| imsi | varchar(15) | NO | PRI | NULL |  |
| msisdn | varchar(46) | YES |  | NULL |  |
| imei | varchar(15) | YES |  | NULL |  |
| imei_sv | varchar(2) | YES |  | NULL |  |
| ms_ps_status | enum('PURGED','NOT_PURGED') | YES |  | PURGED |  |
| rau_tau_timer | int(10) unsigned | YES |  | 120 |  |
| ue_ambr_ul | bigint(20) unsigned | YES |  | 50000000 |  |
| ue_ambr_dl | bigint(20) unsigned | YES |  | 100000000 |  |
| access_restriction | int(10) unsigned | YES |  | 60 |  |
| mme_cap | int(10) unsigned zerofill | YES |  | NULL |  |
| mmeidentity_idmmeidentity | int(11) | NO | PRI | 0 |  |
| key | varbinary(16) | NO |  | 0 |  |
| RFSP-Index | smallint(5) unsigned | NO |  | 1 |  |
| urrp_mme | tinyint(1) | NO |  | 0 |  |
| sqn | bigint(20) unsigned zerofill | NO |  | NULL |  |
| rand | varbinary(16) | NO |  | NULL |  |
| OPc | varbinary(16) | YES |  | NULL |  |

Table 15 SQL Table structure users

TODO column description.

Configuring your HSS

#### Adding your MME

With the help of phpmyadmin: in your database (default is oai_db), in table mmeidentity add your MME:

mmeidentity.idmmeidentity= your MME new key (unique id in 1..N)

mmeidentity.mmehost= your MME fqdn

mmeidentity.mmerealm= your MME realm (should be your MME FQDN without the host name)

mmeidentity.UE-Reachability= 0

#### Adding a user

In table users add your user informations:

user.imsi=IMSI of your USIM.

user.msisdn= MSISDN of your USIM (unused).

users.imei=NULL

users.imei_sv=NULL

users.ms_ps_status=’PURGED’

users.rau_tau_timer=120

users.ue_ambr_ul=50000000

users.ue_ambr_dl=100000000

users.access_restriction=47

users.mme_cap=0

users.mmeidentity_idmmeidentity=’your MME key’

users.RFSP-Index=1

users.urrp_mme=0

users.sqn=’your USIM programmed SQN’

users.rand=0

users.OPc=’the OPc key’ (can be computed by the oai_hss executable when using –k option in build_hss script)

In table pdn allow your user to be served by a APN: Insert a new record and fill all column like other records except for users_imsi column.

Building and running

The EURECOM EPC interact mainly with two other entities: the eNB and the HSS. Depending on the location of the HSS entity, on the same host or not, the building and running options differ:

- When EPC and HSS run on the same host, TCP must be selected as the underlying protocol for DIAMETER on the S6a interface. If EPC and HSS run on separate hosts, SCTP can be selected as the underlying protocol for DIAMETER on the S6a interface. Choosing SCTP instead of TCP makes the network capture of S1-MME traffic easier.

We recommend to follow the step described below, unless you know what you are doing.

MME_GW.

Your EURECOM MME_GW host and your EURECOM HSS host (may be the same host)

Configuration files

Configuration files have to be filled prior to compilation.

Fill OPENAIRCN_DIR/BUILD/EPC/epc.conf.in configuration file.

Building EPC

In a shell go to your OPENAIRCN_DIR/SCRIPTS directory:

If MME_GW and the HSS run on the same host, execute the following commands:

user@mmegwhost:~/openair-cn/SCRIPTS$ ./build_epc --local-hss (optional parameter --clean)

user@mmegwhost:~/openair-cn/SCRIPTS$ ./build_hss --debug –-local-mme (optional parameters: --clean –-operator-key 11111111111111111111111111111111 for example)

Else, execute the following command:

- On MME_GW host:

user@mmegwhost:~/openair-cn/SCRIPTS$ ./build_epc --remote-hss yourhssfqdn (optional parameter --clean)

- On HSS host:

user@hsshost:~/openair-cn/SCRIPTS$ ./build_hss --debug (optional parameters: --clean –-operator-key 11111111111111111111111111111111 for example)

Running EPC

In a shell go to your OPENAIRCN_DIR/SCRIPTS directory:

Execute the following commands:

On MME_GW host or HSS host:

user@host:~/openair-cn/SCRIPTS$ ./run_hss

On MME_GW host:

user@mmegwhost:~/openair-cn/SCRIPTS$ ./run_epc

Have a look at all these executables options (-h option)

Supported scenarios in EPC

E-UTRAN Initial attach

Attach with IMSI

TBD

Attach with GUTI

TBD

Tracking Area Update procedures

TBD

Routing Area Update procedures

Not supported yet.

Service Request procedures

UE triggered Service Request

TBD

Network triggered Service Request

Not supported yet.

S1 Release procedure

TBD

GUTI Reallocation procedure

Detach procedure

UE-Initiated Detach procedure for E-UTRAN

MME-Initiated Detach procedure for E-UTRAN

HSS-Initiated Detach procedure for E-UTRAN

Not supported.

HSS User Profile management function procedure

Not supported.

Bearer deactivation

PDN GW initiated bearer deactivation

Not supported

MME initiated Dedicated Bearer Deactivation

TBD

Intra E-UTRAN handover

Not supported yet

Annex A: Tools for observing, debugging.

Itti_analyzer

Itti_analyzer takes a dump of messages exchanges between the executable (mme_gw or eNB, UE) tasks as input and display these messages in a human readable and comprehensible way. This tool can take as input a file whose content is the XML dump of ITTI messages exchanged between tasks or can act as a server and listen on a socket that a openair executable connects and dump messages in pseudo real-time. Trace messages are also displayed with the tool, but in a second view, that means not interlaced with ITTI messages.

Important:

Prior to use itti_analyzer, you have to instruct the openair-cn executable to dump the ITTI messages to a file with the argument  –K path_to_file.

Installation

In OPENAIRCN_DIR/COMMON/ITTI_ANALYZER directory, execute the following command:

user@host:~ autoreconf –i

user@host:~ ./configure

user@host:~ make

user@host:~ sudo make install

The itti_analyzer executable is now installed on the computer (/usr/local/bin)

Execution

In a shell, execute the following command:

user@host:~ itti_analyzer

The GUI displayed:

Figure 11 ITTI Analyzer main window

For filter selection, please use filters_mme.xml:

Figure 12 ITTI Analizer select filter menu

You can also use options for fastest operations:

user@host:~itti_analyzer -h
Usage: itti_analyser [options]

Options:
  -d DISSECT   write DISSECT file with message types parse details
  -f FILTERS   read filters from FILTERS file
  -h           display this help and exit
  -i IP        set ip address to IP
  -l LEVEL     set log level to LEVEL in the range of 2 to 7
  -m MESSAGES  read messages from MESSAGES file
  -p PORT      set port to PORT

Wireshark/tshark

You can launch wireshark instances on S1 (filter s1ap, gtpu), S6A (filter diameter, if TCP is the undelying protocol, you can select a TCP packet relative to the DIAMETER exchange and the select decode as DIAMETER).

Mscgen

Extract from http://www.mcternan.me.uk/mscgen/: “Mscgen is a small program that parses Message Sequence Chart descriptions and produces PNG, SVG, EPS or server side image maps (ismaps) as the output. Message Sequence Charts (MSCs) are a way of representing entities and interactions over some time period”…” Mscgen aims to provide a simple text language that is clear to create, edit and understand, which can also be transformed into common image formats for display or printing.”…

Openair use mscgen to offer another view of events (SDUs, timers, etc) that happens inside an executable and also (still under development) PDUs exchanged between protocol entities.

Openair HSS do not have the msgen feature.

Important:

Check that mscgen traces are configured for being generated (CFLAG MESSAGE_CHART_GENERATOR set to true in OPENAIRCN_DIR/BUILD/EPC/CMakeLists.template)

You have to instruct the openair mme_gw executable to dump the ITTI messages to a file with the argument  -m path_to_directory. The mscgen files will be located under the specified directory, in a directory containing the time of the generated traces (text and png files).

Example:

Figure 13 Mscgen output example

S1AP scenario replay

This tool is available in the openairinterface5g git repository, branch “Feature-6-fix_test_core_network_with_scenarios”. This branch will be merged in the develop branch as soon as possible.

The aim of this tool is for development, non-regression test, debug purpose, it allows to replay without the help of any eNB(s) or UE(s). a S1AP scenario previously captured as a pcap dumped file.

Overall process

Capture a scenario on S1-C network

Build a generic scenario

Replay scenario

Scenario resource files

Figure 14 Workflow of scenario replay

In order to replay a scenario, a scenario has to be played/captured (step 1), then the captured artefacts have to processed in order to generate a generic scenario (step 2) ready to be replayed on any other testbed (step 3).

The first step consists in capturing a network trace on S1-C network, the second step consists in building a scenario file that is generic, meaning there is no specific testbed references (IP addresses). The last step is the replaying of the scenario on a openair-cn testbed.

Flowchart of step1: Network trace capture on S1-C

The goal of this step is to capture a SCTP/S1AP trace that we want to be able to replay.

The red items in the following flowchart figure are part of the “scenario resource files”.

It is highly recommended to create a dedicated resource directory ($RESOURCE_SCENARIO_DIR) for each created scenario.

eNB(s) conf file

Begin

End

Your scenario, means your UEs interactions with eNB(s)

Start a pcap capture on S1-C

Run HSS

Run eNB(s)

Build HSS

--random false

Run MME /SP-GW

--export-db ‘file_path’

S1 pcap file

HSS database exported file

Figure 15 Flowchart of scenario capture

Step 1: build HSS

The HSS database has first to be configured for not generating randoms in security algorithms, otherwise we will not be able to replay trace coming from UE(s).

The argument that has to be passed to the build_hss script is: --random false

Step 2: run MME-GW

No change here compared to standard case.

Step 3: run HSS

In order to be able to replay the scenario in the same conditions, the initial content of the database has to be saved.

The argument that has to be passed to the run_hss script is:

--export-db $RESOURCE_SCENARIO_DIR/scenario.sql

Step 4: start a pcap-ng capture on S1-C

This step is not automated, you have to start on your own a tool to capture the network traffic on the S1-C network. (you can use wireshark or tshark)

Step 5: start the eNB(s)

The eNB config files will be used later to make a scenario independent of IP addresses.

Step 6: run your scenario

At the end of this process you will have to save in a directory dedicated to your scenario:

- The eNB config files

- The exported database SQL file.

- The pcap-ng file containing all SCTP and S1AP traffic occurred on S1-C network.

Build a generic scenario

Prerequisites:

- Build scenario builder

In openairinterface5g repository (https://twiki.eurecom.fr/twiki/bin/view/OpenAirInterface/GetSources):

#cd openairinterface5g

#source ./oaienv

#cd cmake_targets

#tools/build_test_epc_tools --debug –clean

The executable /usr/local/bin/test_epc_generate_scenario should be generated.

(The executable /usr/local/bin/test_epc_play_scenario should also be generated.)

- In your dedicated scenario resource directory($RESOURCE_SCENARIO_DIR), you should have the three files listed in previous section: the eNB(s) configuration file, the exported database SQL file, the pcap-ng file.

For building a generic scenario, follow the steps described below(assuming the EPC has been built on this host):

# test_epc --test-dir $RESOURCE_SCENARIO_DIR --old-enb-config-file enb.conf --pcap-file my_test.pcap  --build-test

This will generate the files my_test.pdml, my_test.xml in the $RESOURCE_SCENARIO_DIR directory.

my_test.pdml file is the xml dump of the pcap file my_test.pcap, it has served as input for building the generic scenario file my_test.xml.

The following figure shows the inner operations of last test_epc command:

mme_test_s1_pcap2pdml

(usr/local/bin)

start

End

my_test.pcap

my_test.pdml

test_epc_generate_scenario

(/usr/local/bin)

Generic scenario file:

my_test.xml

Figure 16 Build scenario detailed operations

Replay a S1AP generic scenario

Prerequisites:

- Build scenario player (step done in 6.4.3)

In openairinterface5g repository (https://twiki.eurecom.fr/twiki/bin/view/OpenAirInterface/GetSources):

#cd openairinterface5g

#source ./oaienv

#cd cmake_targets

#tools/build_test_epc_tools --debug –clean

The executable /usr/local/bin/test_epc_play_scenario should be generated.

(The executable /usr/local/bin/test_epc_generate_scenario should also be generated.)

- In your dedicated scenario resource directory($RESOURCE_SCENARIO_DIR), you should have almost three mandatory files:

  - the eNB(s) configuration file for your running testbed (may be different from the original enb.conf file if capture testbed and running testbeds are not the same).

  - The exported database SQL file.

  - The generic scenario xml file.

For running a generic scenario, follow the steps described below:

Please note that the eNB(s) emulator “test_epc_play_scenario” can be run exactly like an eNB, from the deployment point of view, it can be run on any EPC host (mme_gw, HSS, or both) or on another host.

# test_epc --test-dir $RESOURCE_SCENARIO_DIR --enb-config-file my_new_enb.conf --scenario-file my_test.xml --run